# Smart Gas Detection and Environment Monitoring System

## Components Required

Arduino UNO

DHT22 Temperature & Humidity Sensor

MQ2 Gas Sensor

16x2 I2C LCD Display

Red LED

Jumper Wires

USB Cable / Power Supply

Breadboard (Optional)

## Pin Connections

| Component Pin | Arduino UNO |
| --- | --- |
| DHT22 VCC | 5V |
| DHT22 GND | GND |
| DHT22 DATA | D7 |
| MQ2 VCC | 5V |
| MQ2 GND | GND |
| MQ2 AO | A0 |
| LCD VCC | 5V |
| LCD GND | GND |
| LCD SDA | A4 |
| LCD SCL | A5 |
| LED + | D8 |
| LED - | GND |

## Working Principle

The Arduino UNO reads temperature and humidity from the DHT22 sensor and gas concentration from the MQ2 sensor. The values are displayed on the LCD. If the gas level is high, the LCD displays 'Gas: DANGER' and the red LED turns ON; otherwise it displays 'Gas: SAFE' and the LED remains OFF.

## Project Link

https://wokwi.com/projects/468286161590235137