Smart Environment Monitoring System Using Arduino UNO

Components Required

Arduino UNO

DHT22 Temperature & Humidity Sensor

MQ2 Gas Sensor

16×2 I2C LCD Display

Jumper Wires

USB Cable / Power Supply

Breadboard (Optional)

Pin Connections

DHT22 Sensor

| DHT22 Pin | Arduino UNO |
| --- | --- |
| VCC | 5V |
| GND | GND |
| DATA | D7 |

MQ2 Gas Sensor

| MQ2 Pin | Arduino UNO |
| --- | --- |
| VCC | 5V |
| GND | GND |
| AO | A0 |

LCD (I2C)

| LCD Pin | Arduino UNO |
| --- | --- |
| VCC | 5V |
| GND | GND |
| SDA | A4 |
| SCL | A5 |

Working Principle

The Arduino UNO reads temperature and humidity values from the DHT22 sensor and gas concentration from the MQ2 sensor. These values are displayed on the 16×2 LCD.

- If the gas value is below the threshold, the LCD displays "Gas: SAFE".

- If the gas value is above the threshold, the LCD displays "Gas: DANGER".

This system helps in monitoring environmental conditions and detecting harmful gas leakage.

Project Link

https://wokwi.com/projects/468283437244832769